ros2 launch kinova_gen3_lite_moveit_config ros2_control_fake.launch.py

- ros2 launch kinova_gen3_lite_moveit_config move_group.launch.py use_fake_hardware:=true robot_ip:=0.0.0.0

ros2 launch kinova_gen3_lite_moveit_config moveit_rviz.launch.py

ros2 launch fyp_task_executor moveit_executor.launch.py  -- Joint control

- ros2 launch my_robot_task_api task_api.launch.py – Task api

ros2 topic pub /pick_now std_msgs/msg/Bool '{data: true}' -1

Terminal 1

source ~/workspace/ros2_kortex_ws/install/setup.bash

export LIBGL_ALWAYS_SOFTWARE=1

export GZ_SIM_RESOURCE_PATH=$(dirname $(ros2 pkg prefix --share kortex_description)):$GZ_SIM_RESOURCE_PATH

export LD_LIBRARY_PATH=/opt/ros/jazzy/lib:$LD_LIBRARY_PATH

gz sim -r ~/workspace/my_robot_task_api/worlds/gen3_world.sdf

Terminal 2

ros2 run robot_state_publisher robot_state_publisher --ros-args -p use_sim_time:=true -p publish_robot_description:=true -p robot_description:="$(cat /tmp/gen3_gz.urdf)"

Terminal 3

ros2 run ros_gz_sim create -name gen3_lite -topic /robot_description

Terminal 4

ros2 run controller_manager spawner joint_state_broadcaster --controller-manager /controller_manager

ros2 run controller_manager spawner joint_trajectory_controller --controller-manager /controller_manager

ros2 run controller_manager spawner gen3_lite_2f_gripper_controller --controller-manager /controller_manager

ros2 run controller_manager spawner gripper_trajectory_controller --controller-manager /controller_manager

Terminal 5

source /opt/ros/jazzy/setup.bash

ros2 run ros_gz_bridge parameter_bridge /clock@rosgraph_msgs/msg/Clock[gz.msgs.Clock

A) Bring up Gazebo + ros2_control (stable)

Terminal 1 — Gazebo

source /opt/ros/jazzy/setup.bash

source ~/workspace/ros2_kortex_ws/install/setup.bash

export LIBGL_ALWAYS_SOFTWARE=1

export GZ_SIM_RESOURCE_PATH=$(dirname $(ros2 pkg prefix --share kortex_description)):$GZ_SIM_RESOURCE_PATH

export LD_LIBRARY_PATH=/opt/ros/jazzy/lib:$LD_LIBRARY_PATH

export GZ_SIM_SYSTEM_PLUGIN_PATH=/opt/ros/jazzy/lib:$GZ_SIM_SYSTEM_PLUGIN_PATH

source /opt/ros/jazzy/setup.bash

source ~/workspace/my_robot_task_api/install/setup.bash  # optional if you need it

# Make sure overlay paths are NOT in your runtime linker

export LD_LIBRARY_PATH=$(echo "$LD_LIBRARY_PATH" | tr ':' '\n' | grep -v "/home/rhenius/workspace/ros2_kortex_ws" | paste -sd ':' -)

export AMENT_PREFIX_PATH=$(echo "$AMENT_PREFIX_PATH" | tr ':' '\n' | grep -v "/home/rhenius/workspace/ros2_kortex_ws" | paste -sd ':' -)

export CMAKE_PREFIX_PATH=$(echo "$CMAKE_PREFIX_PATH" | tr ':' '\n' | grep -v "/home/rhenius/workspace/ros2_kortex_ws" | paste -sd ':' -)

# BUT still give Gazebo access to Kinova description resources (meshes/textures/etc)

export GZ_SIM_RESOURCE_PATH=/home/rhenius/workspace/ros2_kortex_ws/install/kortex_description/share:$GZ_SIM_RESOURCE_PATH

export GZ_SIM_SYSTEM_PLUGIN_PATH=/opt/ros/jazzy/lib:$GZ_SIM_SYSTEM_PLUGIN_PATH

gz sim -r empty.sdf

Terminal 2 — robot_state_publisher (IMPORTANT: publish gen3_final.urdf)

source /opt/ros/jazzy/setup.bash

source ~/workspace/ros2_kortex_ws/install/setup.bash

ros2 run robot_state_publisher robot_state_publisher --ros-args \

-p use_sim_time:=true \

-p publish_robot_description:=true \

-p robot_description:="$(cat /home/rhenius/workspace/my_robot_task_api/urdf/gen3_final.urdf)"

Terminal 3 — spawn model into Gazebo

source /opt/ros/jazzy/setup.bash

source ~/workspace/ros2_kortex_ws/install/setup.bash

ros2 run ros_gz_sim create -name gen3_lite -topic /robot_description

Terminal 4 — spawn controllers

source /opt/ros/jazzy/setup.bash

source ~/workspace/ros2_kortex_ws/install/setup.bash

ros2 run controller_manager spawner joint_state_broadcaster --controller-manager /controller_manager

ros2 run controller_manager spawner joint_trajectory_controller --controller-manager /controller_manager

ros2 run controller_manager spawner gen3_lite_2f_gripper_controller --controller-manager /controller_manager

Quick check (should NOT hang)

ros2 control list_controllers

B) Fix your “arm falls down” issue

If controllers are active but the arm still drops, it’s because no command is holding position yet.

Send a “hold current position” trajectory once:

ros2 topic echo /joint_states --once

Then command something simple (example zeros — adjust if needed):

ros2 action send_goal /joint_trajectory_controller/follow_joint_trajectory control_msgs/action/FollowJointTrajectory "{

trajectory: {

joint_names: ['joint_1','joint_2','joint_3','joint_4','joint_5','joint_6'],

points: [{positions: [0,0,0,0,0,0], time_from_start: {sec: 2}}]

}}"

Once the controller is holding, it should stop falling.

C) Now bring up MoveIt for Gazebo (not fake hardware)

⚠️ Don’t use ros2_control_fake.launch.py or use_fake_hardware:=true anymore.

Use MoveIt with real ros2_control topics (sim time):

Terminal 5 — MoveIt move_group

source /opt/ros/jazzy/setup.bash

source ~/workspace/ros2_kortex_ws/install/setup.bash

ros2 launch kinova_gen3_lite_moveit_config move_group.launch.py \

use_sim_time:=true \

use_fake_hardware:=false \

robot_ip:=0.0.0.0

Terminal 6 — RViz

source /opt/ros/jazzy/setup.bash

source ~/workspace/ros2_kortex_ws/install/setup.bash

ros2 launch kinova_gen3_lite_moveit_config moveit_rviz.launch.py use_sim_time:=true

gz service -s /world/default/create \

--reqtype gz.msgs.EntityFactory \

--reptype gz.msgs.Boolean \

--timeout 300 \

--req 'sdf_filename: "/home/rhenius/workspace/my_robot_task_api/worlds/box.sdf", name: "box"'

ros2 run apriltag_detector apriltag_detector_node --ros-args \ \

-r __node:=apriltag \

-p detector.decimate:=1.0 \

-p detector.refine:=true \

-p family:=36h11 \

-p size:=0.05 \

-r image:=/camera \

-r camera_info:=/camera_info

ros2 run apriltag_ros apriltag_node --ros-args \

--params-file ~/workspace/my_robot_task_api/config/apriltag.yaml \

-r image_rect:=/camera \

-r camera_info:=/camera_info

python ~/workspace/my_robot_task_api/my_robot_task_api/yolo_detector_node.py

python3 ~/workspace/my_robot_task_api/my_robot_task_api/bbox_to_pose_node.py

python3 ~/workspace/my_robot_task_api/my_robot_task_api/llm_planner_node.py

python3 ~/workspace/my_robot_task_api/my_robot_task_api/task_executor_node.py

ros2 topic pub /task_command std_msgs/String "{data: 'pick the ball and place it on the table'}" -1

ros2 run tf2_ros static_transform_publisher 0 0 0 0 0 0 camera_link "gen3_lite/base_link/rgbd_camera"

source /opt/ros/jazzy/setup.bash

ros2 run ros_gz_bridge parameter_bridge \

/rgbd_camera/image@sensor_msgs/msg/Image[gz.msgs.Image \

/rgbd_camera/depth_image@sensor_msgs/msg/Image[gz.msgs.Image \

/rgbd_camera/camera_info@sensor_msgs/msg/CameraInfo[gz.msgs.CameraInfo \

/rgbd_camera/points@sensor_msgs/msg/PointCloud2[gz.msgs.PointCloudPacked

source /opt/ros/jazzy/setup.bash

ros2 run rqt_image_view rqt_image_view

python3 /home/rhenius/workspace/my_robot_task_api/my_robot_task_api/reframe_pointcloud.py